# METHOD OF STATEMENT

Date: 29th July 2024

Description of work

1) To hoist down 1 unit machine from 2nd floor through swing gate opening

2) To hoist up 1 unit machine to 4th floor through swing gate opening

3) To shift out 1 machine from 4th floor to 2nd floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/08/24 (Saturday)

Time of Operation

10:00 am

Location of Operation

Scape Precision Pte Ltd

Blk 9 Kallang Place #04-04, #02-06

Equipment Use

1 x 50 ton mobile crane

1 x 633 FALFINGER lorry loader

Lorry carne

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Mobile crane to hoist up platform to 4th floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Mobile crane to hoist up machine to 4th floor

- Lower down machine onto platform

- Insert rollers/pallet truck under machine bed

- Use of manual pulley to slowly released and to control moving speed of machine

- Shift out machine from platform

- Transport machine into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175