# METHOD OF STATEMENT

Date: 15th Mar 2024

Description of work

To unload cases from 1 x 40ft FR container, uncrate, hoist up machine to clean room

Machine Spec

Packing Size: 520 x 230 x 300 cm/ Wt: 10.035 kgs-Main machine

Packing Size: 362 x 140 x 145 cm/ Wt: 907 kgs –Chip conveyor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/03/24

Time of Operation

10.00am

Location of Operation

#### 9 Tuas Drive 1

#### FuYu Corporation Limited

Equipment Use

1 x 633 lorry loader

Wire/Webbing sling

Shackle

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

P.P.E Required

Safety Helmet

Safety Shoe

Safety Harness

Self Retracting Lifeline

Safety Aspect

1) Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

2) Barricade off work area with safety cones, red/white tape and lifting in operation signage

3) All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            4) Lifting machine and lifting gear with M.O.M certificate

5) Lifting crew shall be composed to the following but not limited by other trained personnel in respect to the number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Riggerman

d. Signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated unloading area

- Setting up of crane with outriggers fully extended and resting on steel plate as base plate

- Hoist down case from container to the ground

- Uncrate case and put aside

- Rigger will attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist up platform together with machine to shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform to building railing/pillar to prevent platform swing outward

- Jack up and insert rollers under machine bed

- Use of manual pulley to slowly released and control moving speed of machine

- Shift out machine from platform

- Position machine to the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Using of forklift to unload cases of machine accessories from trailer

- Uncrate, shift and position accessories to location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175