# METHOD OF STATEMENT

Date: 30th Sep 2024

Description of work

To unload 3 cases of 1 unit machine from low loader, uncrate, hoist up machine to 2nd floor through shutter door opening by using platform and mobile crane

Machine Spec

3 cases of 1 unit Global Advantage 20.33.15 CMM

Case 1: 4600 x 2800 x 745 mm / Wt: 14, 700 kg

Case 2: 3200 x 1400 x 2750 mm / Wt: 1, 000 kg

Case 3: 4740 x 1170 x 960 mm / Wt: 750 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

15/10/24

Time of Operation

10:00 am

Location of Operation

#### 11 Calshot Rd

Singapore Aero Engine Services Pte Ltd

Equipment Use

1 x 100 ton mobile crane

2 x 633 lorry loader

1 x low loader

1 x platform

Wire/webbing sling

Shackle

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Helmet

Safety Shoe

Safety Harness

Self Retracting Lifeline

Safety Aspect

1) Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

2) Barricade off work area with safety cones, red/white tape and lifting in operation signage

3) All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            4) Lifting machine and lifting gear with M.O.M certificate

5) Lifting crew shall be composed to the following but not limited by other trained personnel in respect to the number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Riggerman

e. Signalman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated unloading area

- Setting up of crane with outriggers fully extended and resting on steel plate as base plate

- Hoist down case from low loader to the ground

- Uncrate case and put aside

- Rigger will attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist up platform together with machine to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform to building railing/pillar to prevent platform swing outward

- Jack up and insert rollers under machine bed

- Use of manual pulley to slowly released and control moving speed of machine

- Shift out machine from platform

- To assist Hexagon engineer to install bridge using customer overhead crane

- After installing, position machine into CMM room

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175