# METHOD OF STATEMENT

Date: 29th July 2025

Description of work

To hoist up wooden skid to 2nd floor through shutter door opening by using platform and mobile crane

Spec

Moulding wooden skids

2 unit dim: 6.5m x 2m / Wt: 1, 000 kg each

1 stack dim: 7m x 1.8m / Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

05/08/25

Time of Operation

09:30 am

Location of Operation

#### 88 International Rd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

2 x 40ft trailer

1 x platform

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Helmet

Safety Shoe

Safety Harness

Self Retracting Lifeline

Safety Aspect

1) Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

2) Barricade off work area with safety cones, red/white tape and lifting in operation signage

3) All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            4) Lifting machine and lifting gear with M.O.M certificate

5) Lifting crew shall be composed to the following but not limited by other trained personnel in respect to the number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Riggerman

e. Signalman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated unloading area

- Setting up of crane with outriggers fully extended and resting on steel plate as base plate

- Reverse trailer to near crane

- Hoist and load up wooden skid from trailer onto platform

- Secured skid onto platform with strapping belt

- Mobile crane to hoist up platform together with wooden skid to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform to building railing/pillar to prevent platform swing outward

- Shift out skid from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175