# METHOD OF STATEMENT

Date: 10th Feb 2022

Description of work

To hoist up 1 unit machine to 2nd floor through swing gate opening by using platform and lorry crane

Machine Spec

1 unit Focaseiki TP750 CNC

Dim: 1700 x 2350 x 2400 mm

Wt: 4, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/02/22 (Saturday)

Time of Operation

09:45 am

Location of Operation

#### Blk 20 Woodlands Link #02-05

Accelerated Precision Engineering

Equipment Use

2 x 633 FALFINGER lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of 2 x lorry crane at designated hoisting area for the lift

- 1 x lorry crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175