# METHOD OF STATEMENT

Date: 04th Dec 2023

Description of work

1) To hoist up 2 unit machine to 2nd floor through swing gate opening by using platform and lorry crane

2) Unload 3 unit small item to 2nd floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

13-16/12/23

Time of Operation

09:30 am

Location of Operation

#### Blk 4 Kaki Bukit Ave 1 #02-05

All Industrial Manufacturing Pte Ltd

Equipment Use

2 x lorry loader

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area for the lift

- Barricade off work area

- 1 x lorry crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to the competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Rigger to insert sling to crane hook and secured onto machine rigging point

- Lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert rollers/pallet truck under machine bed

- Shift out machine from platform

- Transport machine into tenant premise

- Position machine at the designated

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175