# METHOD OF STATEMENT

Date: 16th May 2025

Description of work

1) To shift out and hoist up machines from 5th floor to 6th floor through swing gate opening by using platform and mobile/lorry crane

2) To hoist down 3 units machine from 5th floor through swing gate opening by using platform and mobile/lorry crane and load onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate 2 days job:

24/05/25 (Saturday)

26/05/25 (Monday)

Time of Operation

10:00 am

Location of Operation

#### Blk 4 Kaki Bukit Ave 1 #05-01 to #06-02/03

Moveon Technologies Pte Ltd

Equipment Use

2 x 50 ton mobile crane

2 x lorry crane

1 x platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

PPE Required

Safety shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with red/white tape, safety cones and lifting in operation signage

- 1 x 50ton mobile crane to hoist up platform to 5th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab

- Tied back platform to the competent point and secured using 1 x 50ton mobile crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Rigger to insert sling to crane hook and secured onto machine rigging point

- Mobile crane to hoist up machine from 5th floor to 6th floor

- Lower down machine onto platform

- Insert rollers under machine bed

- Use of manual pulley to slowly release and control moving speed of machine

- Shift out machine from platform

- Transport machine into tenant premise

- Position machine at the designated

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175