# METHOD OF STATEMENT

Date: 22nd July 2019

Description of work

1) To hoist up 1 Unit CNC Machining Center to 5th floor through swing gate opening by using platform and mobile crane

2) To shift out and hoist down 1 unit CNC Machining Center from 5th floor through swing gate opening by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27/07/2019

Time of Operation

09:30 am

Location of Operation

#### Blk 5004 A.M.K Ave 5 #05-03

2M Precision Automation Pte Ltd

Equipment Use

2 x 50 ton mobile crane

2 x lorry crane

1 x platform

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 2 x mobile crane to be deploy at designated hoisting area for the lift

- 1 x mobile crane to hoist up platform to 5th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x 50 ton mobile crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x 50 ton mobile crane to hoist up machine to 5th floor

- Lower down machine onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175