# METHOD OF STATEMENT

Date: 19th Feb 2021

Description of work

To reverse cometto to loading bay, jack up engine, insert rollers, shift out from cometto and position at the designated location

Engine Spec

Dim: 580 x 280 x 380 cm

Wt: 25, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22-23/02/2021

Time of Operation

10:00 am

Location of Operation

#### 5 Tukang Innovation Grove

Caterpillar Marine Asia Pacific Pte Ltd

Equipment Use

1 x cometto (with metal plate lay out onto cometto)

Heavy duty rollers

1 set hydraulic jack

1 x forklift

Obstacles

No obstacle on unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- To reverse cometto to loading bay

- Level up cometto with loading bay

- To jack up engine 4” high

- Insert rollers under engine base

- Shift out engine from cometto into factory premise

- Position engine at the designated location

- Unload engine from rollers

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175