# METHOD OF STATEMENT

Date: 10th Sep 2019

Description of work

To unload 1 unit machine and accessories from lorry chassis using forklift, transport and unload at door entrance, shift and position into premise using pallet truck

Machine Spec

1 x Cut P550

Dim: 267 x 223 x 254 cm

Wt: 3, 496 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/09/2019

Time of Operation

12:00 pm

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte

Equipment Use

2 x 633 lorry loader

1 x 4.5 ton forklift

1 set rollers

1 x 5ton pallet truck

Canvas/metal plate (for floor protection)

Obstacles

No obstacle at unloading bay

Barricade operation area to prevent persons who are not involved enter unintentionally

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

# JOB SCOPE

- Transport and deploy lorry loader at designated unloading area

- Lorry loader to hoist down forklift

- Using forklift to fork down machine from lorry chassis

- Transport machine to main door entrance

- Lower down machine onto wooden block

- Insert pallet truck under machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175