# METHOD OF STATEMENT

Date: 04th Feb 2025

Description of work

To hoist up 1 unit tester to 2nd floor through shutter door opening by using platform and mobile/lorry crane

Machine Spec

Via hoisting

1 unit MOW Drop Tester 2

Dim: 205 x 135 x 260 cm / Wt: 1, 000 kg

Via cargo lift

1 unit Granite table

Dim: 170 x 100 x 70 cm

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/02/25 (Sat)

Time of Operation

09:30

Location of Operation

#### 25 Pandan Crescent

Sunningdale Tech Ltd

Equipment Use

1 x 50ton mobile crane

1-2 x 633 lorry loader

1 x platform

Pallet truck

Wire/webbing sling

Shackle

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up crane with outrigger fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Rigger to insert sling to crane hook

- Secured sling to rigging point of platform

- 1 x mobile crane to hoist up platform together with tester to 2nd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure from left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck under tester bed

- Shift out tester from platform

- Position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175