# METHOD OF STATEMENT

Date: 16th July 2024

Description of work

1) To hoist up 4 unit machine to 5th floor by using platform and mobile/lorry crane

2) Lorry crane to hoist and unload smaller machine from lorry chassis, shift and position to 5th floor via cargo lift

Machine Spec

Refer to attached list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate 2 day job

24/08/24 - 26/08/24

Time of Operation

Estimate:

12:00 pm

Location of Operation

#### 25 Pandan Crescent #05-01/02

N & U

Equipment Use

2 x 50ton mobile crane

4-5 x 633 lorry loader

1 x platform

Pallet truck

Rollers

Wire/webbing sling

Hand jack/crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of 2 x mobile crane at designated hoisting area

- Set up crane with outrigger fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- 1 x mobile crane to hoist up platform to 5th floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x mobile crane

- Use strong belt to secure from left and right of platform onto building railing/pillar to prevent platform swing outward

- 1 x mobile crane to hoist up machine to 5th floor

- Lower down machine onto platform

- Insert rollers under machine bed

- Use of manual pulley and slowly release to control moving speed of machine

- Shift out machine from platform

- Position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175