# METHOD OF STATEMENT

Date: 30th Jan 2020

Description of work

To unload 5 cases of 1 unit machine from 1 x 40ft HQ container, uncrate, shift and position into premise at ground floor

Machine Spec

Mill E500

Case 1: 3150 x 2290 x 2540 mm / Wt: 6330 kg

Case 2: 3420 x 1450 x 1610 mm / Wt: 900 kg

Case 3: 1160 x 960 x 1560 mm / Wt: 300 kg

Case 4: 2604 x 1424 x 1712 mm / Wt: 900 kg

Case 5: 950 x 600 x 1100 mm / Wt: 126 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/02/2020

Time of Operation

09:45 am

Location of Operation

#### 10 Dover Drive

SR-B1B SIT @ Dover

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

2pcs 4 x 8ft metal plate (to lay out at door entrance)

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location (back to back with container)

- Pull out case from container onto lorry chassis

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Transport machine into premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175