# METHOD OF STATEMENT

Date: 15th June 2020

Description of work

To hoist down 1 unit machine form lorry chassis to the ground, shift and position into premise

Machine Spec

1 unit Equiptop Grinding

Dim: 170 x 197 x 196 cm

Wt: 1, 375 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

29/06/20

Time of Operation

09:45 am

Location of Operation

#### 10 Dover Drive

SR-B1B SIT @ Dover

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Crane to hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine base

- Transport machine into premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175