# METHOD OF STATEMENT

Date: 03rd May 2021

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, insert pallet truck to machine base, shift and transport into factory premise at ground floor

Machine Spec

1 unit Captura 432

Dim: 134 x 98 x 140 cm

Wt: 375 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/05/21

Time of Operation

09:45 am

Location of Operation

#### 1 Chin Bee Ave

DA/PRO Rubber (S) PL

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secure to machine hoisting point

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck under machine base

- Transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175