# METHOD OF STATEMENT

Date: 16th Dec 2024

Description of work

To hoist down 1 unit robot from lorry chassis to the ground, insert pallet truck under base plate, shift and position into lab at ground floor

Machine Spec

1 unit Robot with base plate

Dim: 1200 x 1200 x 1500 mm

Wt: 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

02/01/25

Time of Operation

Est:

12:00 pm

Location of Operation

#### ITE College West

#### 1 Choa Chu Kang Grove, Blk 1, Lab 5205

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Rigger to insert sling to crane hook

- Secured sling to rigging point of robot base plate

- Hoist up from lorry chassis

- Swing crane and lower down to near ground level

- Insert pallet truck under base plate

- Transport into lab at ground floor

- Position at the designated location

- Unload from pallet truck using crowbar

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175