# METHOD OF STATEMENT

Date: 08th Aug 2023

Description of work

To unload and hoist down equipment from lorry chassis, shift and position to 4th floor via cargo lift

Machine Spec

1 x Granite with stand

Dim: 1000 x 750 x 150 mm

Wt: 350 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/08/23

Time of Operation

Afternoon schedule

Location of Operation

#### 1 Choa Chu Kang Grove

ITE College West

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to hook and secured to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to the ground

- Insert pallet truck under machine stand

- Shift and transport into workshop

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175