# METHOD OF STATEMENT

Date: 03rd Apr 2025

Description of work

To hoist down equipment from lorry chassis, shift and position into factory premise

Spec

1 set Industrial Truck

Dim: 240 x 190 x 250 cm / Wt: 2, 500 kg

Dim: 280 x 140 x 160 cm / Wt: 1, 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/04/25

Time of Operation

10:00 am

Location of Operation

#### 1 Senoko South Rd

#### Sumitomo Bakelite (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to attached sling to crane hook

- Secured sling to rigging point of equipment

- Hoist up equipment from lorry chassis

- Swing crane and lower down equipment to the ground

- Engineer to install equipment

- After install, shift and transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175