# METHOD OF STATEMENT

Date: 07th Dec 2023

Description of work

To unload and hoist down 1 unit machine from lorry chassis, shift and position into premise at ground floor

Machine Spec

1 unit Bandsaw machine

Dim: 230 x 230 x 160 cm

Wt: 1, 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

11-12/12/23

Time of Operation

Between:

09:00 am – 12:00 pm

14:00 pm – 17:00 pm

Location of Operation

#### 10 Hospital Boulevard

Plot H8

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Riggers to attached sling to crane hook and secured to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into premise at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175