# METHOD OF STATEMENT

Date: 23rd Dec 2020

Description of work

1) To hoist down 1 unit machine from lorry chassis to the ground, insert pallet truck to machine base, shift and position into QA room at ground floor

2) To shift out 1 unit Profile Milling from QA room to loading bay, hoist up onto lorry chassis, transport to front loading area, unload and place aside

Machine Spec

1 unit Okamoto ACC 65GX Surface Grinder

Dim: 285 x 218 x 212 cm

Wt: 3, 700 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27/12/2020

Time of Operation

09:45 am

Location of Operation

#### 15 Joo Koon Circle

West Pharmaceutical Services

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Shackle

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Setting up of lorry loader with outriggers fully extended and resting on timber mats/woodblock as base plate

- Insert sling to crane hook and secure to machine hoisting point

- Hoist down machine from lorry chassis to near ground level

- Swing crane to unloading area

- Insert pallet truck under machine base

- Transport machine into QA room at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175