# METHOD OF STATEMENT

Date: 10th June 2024

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into factory premise at ground

Machine Spec

1 unit Manual Lathe

Dim: 227 x 103 x 172 cm

Wt: 1, 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/06/24 (Tue)

Time of Operation

09:30 am

Location of Operation

#### 15 Science Centre Rd

Equipment Use

1 x 633 lorry loader

Rollers

Pallet truck

Wire/webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secured sling to machine rigging point

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175