# METHOD OF STATEMENT

Date: 03rd July 2025

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, insert rollers under machine bed, shift and position into CNC room at ground floor

Machine Spec

1 unit Lathe machine

Dim: 295 x 113 x 182 cm

Wt: 2, 370 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/07/25

Time of Operation

11:30 am

Location of Operation

#### 15 Tuas View Place

John Crane (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Crowbar/woodblock

Metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis and lower down machine to near ground level

- Insert rollers under machine base

- Transport machine into CNC room at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175