# METHOD OF STATEMENT

Date: 11th Jan 2024

Description of work

To hoist and unload 1 unit machine from lorry chassis, insert rollers to machine base, shift and position into production area at ground floor

Machine Spec

1 unit JSSEY NC Lathe

Dim: 315 x 215 x 215 cm

Wt: 2, 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/01/24

Time of Operation

09:45 am

Location of Operation

#### 159 Gul Circle

MC Packaging Pte Ltd

Equipment Use

1 x 633 lorry loader

Wire sling

Rollers/pallet truck

Crowbar/hand jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area for hoisting work

- Setting up of lorry crane with outrigger fully extended

- Rigger to insert required sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and down machine to near ground level

- Insert rollers under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175