# METHOD OF STATEMENT

Date: 04th Oct 2022

Description of work

To unload and hoist down machine and equipment from low loader/lorry chassis, insert rollers to machine base, shift and position into production area  at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

07/10/22

1 unit MPI Booth

4 unit Manipulator

1 set Stress Relieve machine

06/10/22

1 unit Press machine

1 unit Spray Booth + Ducting

1 unit Manipulator

Time of Operation

09:45 am

Location of Operation

#### 16E Tuas Ave 1 #01-61

Hunting Energy Services Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x low loader

Rollers/pallet truck

1 set wire/webbing sling

Shackle

1 x forklift

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Attached sling to crane hook and secured onto rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175