# METHOD OF STATEMENT

Date: 05th June 2025

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, insert pallet truck under machine bed, shift and position into production area at ground floor

Machine Spec

1 unit Okamoto ACC64GX

Dim: 2740 x 2450 x 1850 mm

Wt: 3, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/06/25

Time of Operation

11:00 am

Location of Operation

#### 18 Tractor Rd

NOK Precision Component (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Shackle

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

All submitted certificate shall be made available at all time

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175