# METHOD OF STATEMENT

Date: 13th Dec 2021

Description of work

To unload 2 pallets of equipment from lorry chassis to the ground, using pallet truck to shift and transport to 4th floor via cargo lift

Equipment Spec

Dim: 72” x 48” x 49”

Wt: 900 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/12/21

Time of Operation

09:45 am

Location of Operation

#### 18 Woodlands Loop #04-00

Edmund Optics (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist up and detach equipment from base

- Swing equipment away from base

- Lower down and load onto pallet truck

- Shift and transport to 4th floor via cargo lift

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175