# METHOD OF STATEMENT

Date: 07th July 2023

Description of work

To hoist down 2 skid of 1 unit machine from lorry chassis to the ground, using pallet truck to shift and transport into workshop at ground floor

Machine Spec

RXP 400 DCS High Speed Milling

Skid 1: 230 x 230 x 245 cm

Wt: 3, 200 kg

Skid 2: 200 x 230 x 145 cm

Wt: 650 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/07/23

Time of Operation

09:30 am

Location of Operation

#### 180 A.M.K Ave 8

Nanyang Polytechnic, Blk S, S124

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Unscrew all nuts

- Insert sling to crane hook

- Secured sling to machine rigging point

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into workshop

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175