# METHOD OF STATEMENT

Date: 29th Dec 2025

Description of work

To hoist down 2 cases of 1 set machine from lorry chassis to the ground, uncrate, shift and position into workshop

Machine Spec

1 set Lathe Mill Drill machine

Case 1: 1350 x 760 x 1050 mm

Case 2: 780 x 410 x 600 mm

Total G/N wt: 375 kgs / 345 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/12/25

Time of Operation

10:00 am

Location of Operation

#### ITE College Central

#### 2 A.M.K Dr, Blk H #H1-01

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Rigger to insert sling to crane hook

- Secured sling to rigging point machine

- Hoist up and detach machine from base

- Swing crane and lower down to near ground level

- Insert pallet truck under machine bed

- Transport machine into workshop

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175