# METHOD OF STATEMENT

Date: 23rd July 2025

Description of work

To unload 4 units machine from lorry chassis, insert pallet truck to machine bed, shift and position into production area at ground floor

Machine Spec

Item 1 dim: 1600 x 2000 x 2500 mm / Wt: 300 kg

Item 2 dim: 3400 x 450 x 400 mm / Wt: 400 kg

Item 3 dim: 800 x 500 x 1350 mm / Wt: 200 kg

Item 4 dim: 520 x 600 x 1150 mm / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/07/25

Time of Operation

12:00 pm

Location of Operation

#### 2 Pioneer Sector 3

GE Vernova (S) Services

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Rigger to attached sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175