# METHOD OF STATEMENT

Date: 10th July 2025

Description of work

To unload 1 unit machine and accessories from lorry chassis, insert pallet truck to machine bed, shift and position into production area at ground floor

Machine Spec

1 unit OCT-5540ZA Taiwan Seiki ZNZ EDM Drilling

Dim: 170 x 185 x 235 cm

Wt: 2, 350 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/07/25

Time of Operation

09:30 am

Location of Operation

#### 2 Pioneer Sector 3

GE Repair Solutions (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate case

- Rigger to attached sling to crane hook

- Secure sling to rigging point of machine

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175