# METHOD OF STATEMENT

Date: 02nd Aug 2020

Description of work

Lorry loader to hoist down 1 unit machine from lorry chassis to the ground, using rollers to shift and position into production area at ground floor

Machine Spec

1 unit Manual Lathe

Case Dim: 227 x 130 x 202 cm

Wt: 1, 550 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 09/09/2020

Time of Operation

09:45 am

Location of Operation

#### 21 Loyang Crescent

Safran Landing Systems Services (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck/rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate case

- Hoist and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers under machine base

- Shift and transport machine into production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175