# METHOD OF STATEMENT

Date: 11th Nov 2025

Description of work

To hoist up Dumpster onto dumpster frame, insert rollers, shift and position to designated location

Machine Spec

1 unit Dumpster

Wt: 4, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/11/25

Time of Operation

09:00 am

Location of Operation

#### 28 Kranji Loop, Kranji Green #01-02

Chung Ah

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling

Hand jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to insert sling to lorry crane hook

- Secured sling to rigging point of dumpster

- Hoist up dumpster and load onto dumpster frame

- Insert rollers under frame

- Shift and position dumpster at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175