# METHOD OF STATEMENT

Date: 25th Aug 2025

Description of work

To hoist down 5 units machine from lorry chassis to the ground, insert rollers under machine bed, shift and position into production at ground floor

Machine Spec

1 unit PSG C-60100 AHR Surface Grinder / Wt: 7, 500 kg

1 unit Youji YV800 ATC / Wt: 20, 500 kg

1 unit Hwacheon VT650R / Wt: 10, 100 kg

1 unit Hwacheon L1-1500 / Wt: 16, 300 kg

1 unit VL1600 / Wt: 8, 740 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

17-18/09/25 – move in (PSG C-60100, Youji YV800, Hwacheon VT650R)

23-24/09/25 – move in (Hwacheon L1-1500, VL1600)

Time of Operation

Est: 13:00 pm

Location of Operation

#### 2A Changi North St 2

ETLA Ltd

Equipment Use

110 ton lorry loader

2 x 633 lorry loader

1 x low-bed

Rollers/pallet truck

Wire/webbing sling

Hydraulic/hand jack

Woodblock

Metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Transport machine into production at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175