# METHOD OF STATEMENT

Date: 30th Jan 2024

Description of work

To hoist down machine and accessories from 1 x 40ft FR, 2 x 40ft HQ and 1 x 20GP container to the ground, uncrate, shift and position into production area at ground floor

Machine Spec

1 unit Nigata machine

Dim: 5.73 x 3.3 x 3.93

Wt: 20, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/01/24   -   to unload and move in machine accessories

01/02/24   -   to unload and move in main machine

Time of Operation

Afternoon schedule

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 110ton lorry loader

2-3 x 633 lorry loader

1 x low loader

Rollers/pallet truck

Wire sling/shackle

Hydraulic jack/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- To reverse low loader to new crane

- Rigger will insert sling to crane hook

- Secure sling to machine rigging point

- Hoist up machine from low loader chassis

- Move out low loader

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175