# METHOD OF STATEMENT

Date: 25th Apr 2024

Description of work

To shift and position machine from outside factory into production area at ground floor, using factory overhead crane to hoist up onto foundation

Machine Spec

1 unit Nigata machine

Dim: 5.73 x 3.3 x 3.93

Wt: 20, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/04/24

Time of Operation

09:45 am

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

Rollers/pallet truck

Webbing sling/shackle

Hydraulic jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- To jack up machine to 4” high using hydraulic jack

- Insert rollers under machine bed

- Transport machine into production area at ground floor

- Using factory overhead crane to hoist up machine

- Position machine onto foundation

- Unload machine and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175