# METHOD OF STATEMENT

Date: 19th July 2024

Description of work

1) To hoist down 1 unit machine from lorry chassis, shift and position to 3rd floor via cargo lift

2) To shift out 1 unit machine from 3rd floor and load up onto lorry chassis

Machine Spec

Move in

1 unit 3HP Milling

Wt: 1, 300 kg

Move out

1 unit small Bench Drilling

Wt: 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/07/24

Time of Operation

12:00 pm

Location of Operation

#### NUS

3 Science Dr 2, Blk S15 #03-18

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine to 3rd floor via cargo lift

- Position at the designated location

- To shift out 1 unit small machine to loading area

- Hoist up and load onto lorry chassis

- To transport and deliver to other location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175