# METHOD OF STATEMENT

Date: 29th Nov 2021

Description of work

To unload 2 packages of 1 unit machine from lorry chassis to the ground, uncrate, using pallet truck and transport to 4th floor via passenger lift

Machine Spec

Datron CNC High Speed Milling

Package 1

Dim: 148 x 103 x 216 cm

Wt: 870 kg

Package 2

Dim: 143 x 128 x 147 cm

Wt: 322 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/12//21

Time of Operation

09:45 am

Location of Operation

#### 3 Sentosa Gateway – St James Power Station

Blk A, #04

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

3mm plywood (to protect ground floor flooring)

4pcs of 12mm plywood (to protect 4th floor raise floor)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down and load onto pallet truck

- Shift and transport to 4th floor via passenger lift

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175