# METHOD OF STATEMENT

Date: 15th Aug 2025

Description of work

To hoist down equipment from lorry chassis to the ground, insert pallet truck, shift and position into production

Spec

1 x Robot / Wt: 500 kg

Table / Wt: 200 kg

Control panel / Wt: 150 kg

Base 1: 150 kg

Base 2 / Wt: 100 kg

2 x Trolley / Wt: 150 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/08/25

Time of Operation

Est: 12:00 pm

Location of Operation

#### 3 Tuas Loop

Owen Corning

Equipment Use

1 x 633 lorry loader

Pallet truck

1 x manual lifter

Webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Rigger to insert sling to crane hook

- Secured sling to rigging point of equipment

- Hoist up equipment from lorry chassis

- Swing crane and lower down to near ground level

- Insert pallet truck under equipment

- Transport into production at ground floor

- Position at the designated location

- Using of manual lifter to lift up table onto machine

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175