# METHOD OF STATEMENT

Date: 04th July 2023

Description of work

To unload and hoist down machineries from lorry/low loader chassis, shift and position into tenant production area

Machine Spec

Refer to attached machine list

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

Estimate 2/3 day job

25-27/07/23

Time of Operation

10:00 am

Location of Operation

#### 30 Kranji Loop #02-11

TimMac @ Kranji, Blk B

Equipment Use

3-4 x lorry loader

1 x low loader

Rollers

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secured to machine rigging point

- Hoist up machine from low loader/lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into tenant production area

- Position machine at the designated location

- Repeat process till all machine are well positioned

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175