# METHOD OF STATEMENT

Date: 26th July 2023

Description of work

To unload 1 unit machine from lorry chassis, shift and position into production area at ground floor

Machine Spec

1 unit Okamoto 350II Grinder

Dim: 125 x 153 x 205 cm

Wt: 940 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

15/08/23

Time of Operation

12:00 pm

Location of Operation

#### 30 Tuas Ave 2

Becton Dickinson

Equipment Use

1 x 633 lorry loader

Pallet truck

Crowbar/woodblock

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to attached sling to crane hook and secured to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175