# METHOD OF STATEMENT

Date: 18th Mar 2025

Description of work

To unload 1 unit machine from lorry chassis, shift and position to the designated location

Machine Spec

1 unit machine

Dim: 120 x 80 x 80 cm / Wt: 130 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/03/25

Time of Operation

Est: 11:30 am

Location of Operation

#### 4 Fusionopolis Way, Fusionopolis 2 RM-L7-001

#### Singapore Institute of Manufacturing Technology

Equipment Use

1 x 633 lorry loader

1 x manual lifter

Pallet truck

Webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport to 7th floor via cargo

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175