# METHOD OF STATEMENT

Date: 16th Dec 2024

Description of work

To hoist down 4 cases from lorry chassis, shift and position into production area at ground floor without uncrating

Machine Spec

1 x Nikon SLM280 Printer machine with accessories

Case 1: 210 x 145 x 225 cm / Wt 1, 800 kg

Case 2: 122 x 105 x 195 cm

Case 3: 130 x 130 x 135 cm

Case 4: 130 x 95 x 225 cm

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/12/24

Time of Operation

Est:

14:00 pm

Location of Operation

#### 503 Paya Lebar Rd

ST Aerospace Engines Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Attached sling to crane hook and secured to rigging point of case

- Hoist up case from lorry chassis

- Swing crane and lower down case to the ground

- Insert pallet truck under case

- Shift and transport into production area at ground floor

- Position case at the designated location without uncrating

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175