# METHOD OF STATEMENT

Date: 19th Dec 2024

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into production area at ground floor

Machine Spec

1 unit CH-430x2000 Lathe machine

Wt: 2, 450 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/12/24

Time of Operation

10:00 am

Location of Operation

#### 55 Loyang Dr

Asian Surface Technologies Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set webbing sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport into production area at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175