# METHOD OF STATEMENT

Date: 14th Feb 2024

Description of work

To hoist down 4 packages from lorry chassis to the ground, shift and position into factory premise at ground floor

Spec

Package 1: 1900 x 2300 x 1800 mm / Wt: 1, 150 kg

Package 2: 2700 x 2300 x 1400 mm / Wt: 950 kg

Package 3: 2700 x 2300 x 1000 mm / Wt: 950 kg

Package 4: 3300 x 2300 x 800 mm / Wt: 950 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/02/24

Time of Operation

09:40 am

Location of Operation

#### 565 Transit Rd

#### Nee Soon Camp

Equipment Use

2-3 x 633 lorry loader

Pallet truck

Wire/webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secured sling onto pallet

- Hoist up from lorry chassis

- Swing crane and lower down to the ground

- Insert pallet truck under pallet

- Transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175