# METHOD OF STATEMENT

Reference No: EW/Q/1448/22

Date: 04th July 2023

Description of work

To unload 3 item of 1 unit machine from lorry to the ground, insert rollers to machine bed, shift and position into lab at ground floor

Machine Spec

1 unit YCM-NSV66A YCM Vertical Machining Center

Dim: 333 x 227 x 257 cm

Wt: 6, 310 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/07/23

Time of Operation

11:00 am

Location of Operation

#### 8 Somapah Rd

SUTD, Blk 1 #01

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended

- Insert sling to crane hook and secure to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Transport machine into lab at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175