# METHOD OF STATEMENT

Date: 03rd Oct 2023

Description of work

To unload and hoist down 3 unit equipment from lorry chassis, shift and position into production at ground floor

Equipment Spec

1 unit Optic centric

Dim: 245 x 155 x 205 cm / Wt: 180 kg

1 unit Conveyor link up

Dim: 98 x 42 x 185 cm / Wt: 75 kg

1 unit Output buffer

Dim: 162 x 88 x 188 cm / Wt: 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/10/23

Time of Operation

11:00 am

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to crane hook and secured to rigging point of equipment

- Hoist up equipment

- Swing crane and lower down equipment from lorry chassis to the ground

- Release all sling from crane hook

- Insert pallet truck under equipment base

- Shift and transport equipment into production at ground floor

- Position equipment at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175