# METHOD OF STATEMENT

Date: 09th May 2023

Description of work

To unload and hoist down 2 cases of 1 unit machine from lorry chassis, shift and position into production at ground floor

Machine Spec

48” Air Bearing Polishing machine

Case 1: 208 x 228 x 179 cm / Wt: 2, 640 kg

Case 2: 76 x 66 x 129 cm / Wt: 150 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24-25/05/23

Time of Operation

09:45 am

Location of Operation

#### 8 Tractor Rd

Excelitas Technologies (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate case

- Insert sling to crane hook and secured to rigging point of machine

- Hoist and detach machine from base

- Swing crane and lower down machine from lorry chassis to the ground

- Release all sling from crane hook

- Insert pallet truck under machine base

- Shift and transport machine into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175