# METHOD OF STATEMENT

Date: 01st Dec 2022

Description of work

To unload and hoist down 7 packages of 1 unit machine from low bed/lorry chassis, insert rollers to machine base, shift and position into production area  at ground floor

Machine Spec

UNISHEAR SYSTEM W/SR400S

386 x 229 x 211 cm / Wt: 5, 300 kg

249 x 196 x 254 cm / Wt: 2, 400 kg

135 x 49 x 41 cm / Wt: 140 kg

122 x 82 x 74 cm / Wt: 680 kg

122 x 72 x 244 cm / Wt: 370 kg

125 x 125 x 79 cm / Wt: 840 kg

84 x 69 x 191 cm / Wt: 150 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 27-31/12/22

Time of Operation

10:00 am

Location of Operation

#### 9 Benoi Sector

Apple Recycling

Equipment Use

633 lorry loader

1 x low bed

1 x forklift

Rollers/pallet truck

1 set wire/webbing sling

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Reverse low bed to near crane

- Attached required sling to crane hook and secured to rigging point of machine

- Hoist up machine from low bed chassis

- Swing crane and lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into production area at ground floor

- Position machine at the designated location

- Use of forklift to unload smaller item from lorry chassis

- Shift and position into production

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175