# METHOD OF STATEMENT

Date: 02nd Mar 2023

Description of work

To unload and hoist down machine from lorry chassis, shift and position into customer production area

Machine Spec

1 unit Matrix Measuring machine

Dim: 180 x 85 x 165 cm / Wt: 400 kg

1 unit Micro Vu Measuring machine

Dim: 110 x 80 x 120 cm / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/03/23  –  to deliver Matrix Measuring machine

17-28/04/23  –  to deliver Micro Vu Measuring machine

Time of Operation

12:00 am

Location of Operation

#### 9 Pioneer View

Resmed Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling to machine rigging point

- Hoist down machine from lorry chassis

- Lower down machine to near ground level

- Insert pallet truck under machine base

- Shift and transport machine into customer production area

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175