# METHOD OF STATEMENT

Date: 05th June 2024

Description of work

To unload and hoist down machines and accessories from lorry chassis by using back lane of building, shift and position into tenant production area at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

2 day job

19-20/07/24

Time of Operation

12:00 pm

Location of Operation

#### Blk 1004 Toa Payoh North #01-16/17

MJ Precision Tools Pte Ltd

Equipment Use

4-5 x lorry loader

Pallet truck/rollers

Wire sling

Webbing sling

1 x platform (to lay out for machine pathway)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back lane of building)

- Barricade off the work area

- Hoist down platform and lay out in front of tenant door

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine onto platform

- Insert pallet truck/rollers under machine base

- Shift and transport machine into tenant premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175